TYPICAL MAINTENANCE ACTIVITIES FOR DETENTION PONDS

Adapted from the Georgia Stormwater Management Manual

| Activity | Schedule |
| --- | --- |
| - Clean and remove debris from inlet and outlet structures. - Clear* exterior and interior slopes of dam and easements to pond. Trees are not allowed on the dam or in drainage easements. Note that healthy ground cover is necessary to prevent erosion. | Monthly / Routine |
| - Remove debris from basin surface to minimize outlet clogging and improve aesthetics. - If wetland components are included, inspect for invasive vegetation. | Semiannual Inspection and following significant storm events |
| - Inspect for damage, paying particular attention to the control structure. - Inspect for invasive vegetation. - Monitor for sediment accumulation in the facility and forebay (flow area into pond). - Examine to ensure that inlet and outlet devices are free of debris and are operational. For wet detention ponds: - Inspect for signs of eutrophica conditions in wet ponds. - Perform wetland plant management and harvesting. | Annual Inspection |
| - Repair undercut or eroded areas. - Seed or sod to restore dead or damaged ground cover. - Remove litter/debris. - Perform structural repairs to inlets and outlet. | As needed based on inspection |
| - Remove accumulated sediment from the forebay. | 5 to 7 years or after 50% of the total forebay capacity has been lost. |
| - Monitor sediment accumulation, and remove sediment when the pond volume has become reduced significantly, or the pond becomes eutrophic. | 10-20 years or after 25% of the permanent pool volume has been lost. |

- Having waters rich in mineral and organic nutrients that promote a proliferation of plant life, especially algae, which reduces the dissolved oxygen content and often causes the extinction of other organisms. Control with nutrient management.

* Periodic mowing of the pond area is only required along access and outfall easements and the embankments, including the outside of the dam. The remaining areas can be managed as a meadow (mowing every other year) or forest.